Heterocyclic lead(II) thioureato complexes as single-source precursors for the aerosol assisted chemical vapour deposition of PbS thin films

Kevin I. Y. Ketchemena,b, Sixberth Mlowea, Linda D. Nyamenb, Ahmed A. Abouda,c, Matthew P. Akermand, Peter T. Ndifonb*, Paul O’Briene* and Neerish Revaprasadua*

aDepartment of Chemistry, University of Zululand, Private Bag X1001, KwaDlangezwa 3880, South Africa. email: RevaprasaduN@unizulu.ac.za

bDepartment of Inorganic Chemistry, University of Yaoundé I, P.O. Box 812, Yaoundé, Cameroon.

c Department of Physics, Faculty of Science, Beni-Suef University, Beni-Suef 62514, Egypt.

dSchool of Chemistry and Physics, University of KwaZulu-Natal, Private Bag X01, Scottsville, Pietermaritzburg, 3209, South Africa.

eThe School of Chemistry and The School of Materials, The University of Manchester Oxford Road, Manchester M13 9PL, UK.

ABSTRACT

The lead N’-benzoylthioureato complexes of N-morpholine (1) and N-pyrrolidine (2) were synthesized and characterized by elemental analysis, Fourier Transform Infra-Red (FT-IR) spectroscopy, proton nuclear magnetic resonance (1H nmr) and thermogravimetric analyses (TGA). The X-ray single crystal structure of complex (1) was determined. The compounds were both used as single source precursors to deposit PbS films on glass substrates at 350, 400 and 450 °C using aerosol assisted chemical vapour deposition (AACVD) The surface morphology of PbS films were determined by scanning electron microscopy (SEM), the crystalline phases established by powder X-ray diffraction (p-XRD) analyses and composition by energy dispersive X-ray spectroscopy (EDX). The particle sizes were found to range between 82 – 85 nm from complex (1) and 70 – 105 nm for (2) The PbS films were studied by Near Infra-red (NIR) UV–Vis spectroscopy, band gap ranging from 1.46 – 1.55 eV were observed.

Keywords: Single source precursors, lead sulfide, AACVD, thin films, optical properties.